COMPUTER NETWORKS

ASSIGNMENT # 1

QUIC Analysis

Submitted by: Izza Iqbal

Registration no: B23F0317AI119

Submitted to: Dr Adnan Iqbal

Department: BSAI Red

Date: 22nd Sep 2025

QUIC Website Analysis:

I have attach the file to capture QUIC, in which I did the following steps

Steps:

- Capturing QUIC Traffic

- Generating QUIC Traffic

- Analyzing QUIC Packets

Questions:

- QUIC Website Name:

- Examine initial QUIC handshake packets

- The protocol column shows "QUIC"

- Using UDP port 443

- Multiple QUIC packet types visible (Initial, Handshake, Protected Payload)

- Initial QUIC Handshake

  Find Initial packet: Look for QUIC packet with "Initial" packet type

  Extract information: Connection ID, version information

I have,

- Initial packets captured

- Connection ID extracted

- Version information

- QUIC protocol analysis

- TLS ClientHello in QUIC:

  Location: Inside QUIC Initial packet, find embedded TLS ClientHello

  Method: Expand packet details to find TLS handshake data

- QUIC Version Analysis:

  Check Version field: In QUIC header, note the version being used

  Compare: Note differences from traditional TCP-based connections

Version Found: QUIC Version 1 (0x00000001)

- This is the standardized QUIC version defined in RFC 9000

- Represents the first official release of QUIC protocol

Key Differences from Traditional TCP-based Connections:

Transport Protocol:

- QUIC: Built on UDP (User Datagram Protocol)

- TCP: Uses Transmission Control Protocol

Connection Establishment:

- QUIC: 0-RTT or 1-RTT connection establishment (faster)

- TCP: Requires 3-way handshake + separate TLS handshake (slower)

Security Integration:

- QUIC: TLS 1.3 built into the protocol (mandatory encryption)

- TCP: TLS is a separate layer on top of TCP

Stream Multiplexing:

- QUIC: Native multiplexing with independent streams (no head-of-line blocking)

- TCP: Single byte stream (head-of-line blocking affects all data)

Connection Migration:

- QUIC: Supports seamless connection migration across network changes

- TCP: Connection breaks when IP address or port changes

Congestion Control:

- QUIC: Modern congestion control algorithms built-in

- TCP: Legacy congestion control mechanisms

Evidence: Screenshot shows "Version: 1 (0x00000001)" in QUIC connection information, confirming use of standardized QUIC v1 protocol.

This analysis shows how QUIC represents a modern evolution beyond traditional TCP-based connections, offering improved performance and built-in security.

#### 5. 0-RTT vs 1-RTT Analysis

- Examine packet flow: Check if 0-RTT data is present

- Determine: Whether connection uses 0-RTT or 1-RTT establishment

0-RTT vs 1-RTT Analysis:

Connection Type: 1-RTT establishment

Evidence:

- Initial packets followed by Handshake packets

- Protected Payload (application data) appears after handshake completion

- No early data transmission observed

- Full cryptographic handshake completed before data exchange

#### 6. HTTP/3 vs HTTP/2 Comparison

- Analyze differences: Compare QUIC-based HTTP/3 with TCP-based HTTP/2

- Note differences: Connection establishment, multiplexing, packet structure

HTTP/3 vs HTTP/2 Comparison:

Connection Establishment:

- HTTP/3: QUIC 0-RTT/1-RTT handshake (faster)

- HTTP/2: TCP 3-way + TLS handshake (slower)

Multiplexing:

- HTTP/3: Independent QUIC streams, no head-of-line blocking

- HTTP/2: TCP-level head-of-line blocking affects all streams

Packet Structure:

- HTTP/3: QUIC frames containing HTTP data

- HTTP/2: TCP segments containing HTTP/2 frames

Transport Protocol:

- HTTP/3: UDP-based QUIC

- HTTP/2: TCP-based

Security:

- HTTP/3: Mandatory built-in TLS 1.3

- HTTP/2: Optional TLS layer

Evidence from Capture:

- QUIC protocol visible in packet list

- UDP port 443 usage

- CRYPTO frames containing TLS data

- Stream multiplexing within single QUIC connection